SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

SECTION 1. IDENTIFICATION

Product name : SAPPHIRE DRESSING

Product code : SAPP0001

Manufacturer or supplier's details

Company name of supplier : Mi 6:8

Address : UNIT 4 38 ADELAIDE ST N

Email Address : MI68JEFF@GMAIL.COM

Telephone : 519 902 6227

Emergency telephone num-: 1-800-424-9300 / 1-703-741-5970

ber

Recommended use of the chemical and restrictions on use

Recommended use : TIRE SHINE / DRESSING

Restrictions on use : Use only outdoors or in a well-ventilated area.

SECTION 2. HAZARDS IDENTIFICATION

GHS classification in accordance with 29 CFR 1910.1200

Flammable liquids : Category 2

Skin irritation : Category 2

Specific target organ toxicity : Category 3 (Central nervous system)

- single exposure

Aspiration hazard : Category 1

GHS label elements

Hazard pictograms :

Signal word : Danger

Hazard statements : Highly flammable liquid and vapour.

May be fatal if swallowed and enters airways.

Causes skin irritation.

1 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

May cause drowsiness or dizziness.

Precautionary statements :

Prevention:

Keep away from heat/sparks/open flames/hot surfaces. No

smoking.

Keep container tightly closed.

Ground/bond container and receiving equipment.

Use explosion-proof electrical/ ventilating/ lighting equipment.

Use only non-sparking tools.

Take precautionary measures against static discharge.

Avoid breathing dust/ fume/ gas/ mist/ vapours/ spray.

Wash skin thoroughly after handling.

Use only outdoors or in a well-ventilated area.

Wear protective gloves/ eye protection/ face protection.

Response:

IF SWALLOWED: Immediately call a POISON CENTER/doctor.

IF ON SKIN (or hair): Take off immediately all contaminated

clothing. Rinse skin with water/shower.

IF INHALED: Remove person to fresh air and keep comfortable

for breathing. Call a POISON CENTER/doctor if you feel unwell.

Do NOT induce vomiting.

If skin irritation occurs: Get medical advice/ attention.

Take off contaminated clothing and wash before reuse.

In case of fire: Use dry sand, dry chemical or alcohol-resistant

foam to extinguish.

Storage:

Store in a well-ventilated place. Keep container tightly closed.

Store in a well-ventilated place. Keep cool.

Store locked up.

Disposal:

Dispose of contents/ container to an approved waste disposal

plant.

Other hazards

Static-accumulating flammable liquid.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Substance / Mixture :  Mixture

Hazardous components

Chemical name CAS-No. Concentration (% w/w)

Heptane, branched, cyclic and linear 426260-76-6 >= 70 - <  90

n-Heptane 142-82-5 >= 1 - <  5

Any concentration shown as a range is to protect confidentiality or is due to batch variation.

SECTION 4. FIRST AID MEASURES

2 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

General advice : Move out of dangerous area.

Show this safety data sheet to the doctor in attendance.

Symptoms of poisoning may appear several hours later.

Do not leave the victim unattended.

If inhaled : Move to fresh air.

Consult a physician after significant exposure.

If unconscious, place in recovery position and seek medical

advice.

If symptoms persist, call a physician.

In case of skin contact : If on clothes, remove clothes.

Remove contaminated clothing. If irritation develops, get med-

ical attention.

If on skin, rinse well with water.

Wash contaminated clothing before re-use.

If skin irritation persists, call a physician.

In case of eye contact : Flush eyes with water as a precaution.

Remove contact lenses.

Protect unharmed eye.

Keep eye wide open while rinsing.

If swallowed : Obtain medical attention.

Do NOT induce vomiting.

Do not give milk or alcoholic beverages.

Never give anything by mouth to an unconscious person.

If symptoms persist, call a physician.

Most important symptoms : May be fatal if swallowed and enters airways.

and effects, both acute and Causes skin irritation.

delayed May cause drowsiness or dizziness.

SECTION 5. FIREFIGHTING MEASURES

Suitable extinguishing media : Water spray

Carbon dioxide (CO2)

Dry chemical

Alcohol-resistant foam

Unsuitable extinguishing : High volume water jet

media

Specific hazards during fire-: Do not allow run-off from fire fighting to enter drains or water

fighting courses.

Hazardous combustion prod-:  Carbon oxides

ucts Silicon oxides

Specific extinguishing meth-: Product is compatible with standard fire-fighting agents.

ods

Further information : Use extinguishing measures that are appropriate to local cir-

cumstances and the surrounding environment.

3 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

Fire residues and contaminated fire extinguishing water must

be disposed of in accordance with local regulations.

Use a water spray to cool fully closed containers.

Special protective equipment : In the event of fire, wear self-contained breathing apparatus.

for firefighters

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal precautions, protec-: Use personal protective equipment.

tive equipment and emer-Remove all sources of ignition.

gency procedures Ensure adequate ventilation.

Avoid breathing dust.

Beware of vapours accumulating to form explosive concentra-

tions. Vapours can accumulate in low areas.

Evacuate personnel to safe areas.

Persons not wearing protective equipment should be excluded

from area of spill until clean-up has been completed.

Environmental precautions : Prevent further leakage or spillage if safe to do so.

Prevent product from entering drains.

Do not flush into surface water or sanitary sewer system.

If the product contaminates rivers and lakes or drains inform

respective authorities.

Methods and materials for : Contain spillage, and then collect with non-combustible ab-

containment and cleaning up sorbent material, (e.g. sand, earth, diatomaceous earth, ver-

miculite) and place in container for disposal according to local

/ national regulations (see section 13).

SECTION 7. HANDLING AND STORAGE

Advice on protection against :  Take necessary action to avoid static electricity discharge

fire and explosion (which might cause ignition of organic vapours).

Electrically bond and ground all containers, personnel and

equipment before transfer or use of material.  Special precau-

tions may be necessary to dissipate static electricity for non-

conductive containers. Use proper bonding and grounding

during product transfer as described in National Fire Protec-

tion Association document NFPA 77.

Keep away from open flames, hot surfaces and sources of

ignition.

Use only explosion-proof equipment.

Do not spray on a naked flame or any incandescent material.

Advice on safe handling : Open drum carefully as content may be under pressure.

Avoid formation of aerosol.

Provide sufficient air exchange and/or exhaust in work rooms.

Do not breathe vapours/dust.

Do not smoke.

Take precautionary measures against static discharges.

Avoid contact with skin and eyes.

Dispose of rinse water in accordance with local and national

4 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

regulations.

Container hazardous when empty.

Smoking, eating and drinking should be prohibited in the ap-

plication area.

For personal protection see section 8.

Conditions for safe storage : Keep container tightly closed in a dry and well-ventilated

place.

Containers which are opened must be carefully resealed and

kept upright to prevent leakage.

Observe label precautions.

No smoking.

Further information on stor-:  No decomposition if stored and applied as directed.

age stability

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Components with workplace control parameters

Components CAS-No. Value type Control parame-Basis

(Form of ters / Permissible

exposure) concentration

Heptane, branched, cyclic and 426260-76-6 TWA 400 ppm ACGIH

linear

STEL 500 ppm ACGIH

n-Heptane 142-82-5 TWA 85 ppm NIOSH REL

350 mg/m3

C 440 ppm NIOSH REL

1,800 mg/m3

TWA 500 ppm OSHA Z-1

2,000 mg/m3

TWA 400 ppm OSHA P0

1,600 mg/m3

STEL 500 ppm OSHA P0

2,000 mg/m3

TWA 400 ppm ACGIH

STEL 500 ppm ACGIH

Engineering measures : Provide sufficient mechanical (general and/or local exhaust)

ventilation to maintain exposure below exposure guidelines (if

applicable) or below levels that cause known, suspected or

apparent adverse effects.

Personal protective equipment

Respiratory protection : In the case of vapour formation use a respirator with an ap-

proved filter.

Hand protection

Remarks : Wear resistant gloves (consult your safety equipment suppli-

er). The suitability for a specific workplace should be dis-

cussed with the producers of the protective gloves. Discard

5 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

gloves that show tears, pinholes, or signs of wear.

Eye protection : Not required under normal conditions of use.  Wear splash-

proof safety goggles if material could be misted or splashed

into eyes.

Skin and body protection : Choose body protection according to the amount and con-

centration of the dangerous substance at the work place.

Wear as appropriate:

Impervious clothing

Flame-resistant clothing

Safety shoes

Hygiene measures : Handle in accordance with good industrial hygiene and safety

practice.

When using do not smoke.

When using do not eat or drink.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance : liquid

Colour :  purple

Odour :  fruity

Odour Threshold :  not determined

pH : not determined

Melting point/freezing point : not determined

Boiling point/boiling range : > 65 °C

(1,013 hPa)

The value is calculated

Flash point : < 20.5 °C

Method: Pensky-Martens closed cup

Evaporation rate :  not determined

Flammability (solid, gas) : not determined

Flammability (liquids) : Static-accumulating flammable liquid.

Self-ignition : 450 °C

Upper explosion limit / Upper : 7 %(V)

flammability limit The value is calculated

Lower explosion limit / Lower : 0.6 %(V)

flammability limit The value is calculated

Vapour pressure : 35 hPa (25 °C)

The value is calculated

6 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

Relative vapour density : not determined

Density : 0.7488 g/cm3

Solubility(ies)

Water solubility : partly soluble

Partition coefficient: n-: No data available

octanol/water

Decomposition temperature :  not determined

Viscosity

Viscosity, dynamic : not determined

Viscosity, kinematic : 18.6 mm2/s (40 °C)

Oxidizing properties : No data available

Molecular weight : No data available

SECTION 10. STABILITY AND REACTIVITY

Reactivity :  No dangerous reaction known under conditions of normal use.

No decomposition if stored and applied as directed.

Chemical stability :  Stable under normal conditions.

No decomposition if stored and applied as directed.

Possibility of hazardous reac-: No decomposition if stored and applied as directed.

tions Vapours may form explosive mixture with air.

Conditions to avoid : Heat, flames and sparks.

Incompatible materials :  Strong oxidizing agents

Hazardous decomposition : Carbon oxides

products Silicon oxides

SECTION 11. TOXICOLOGICAL INFORMATION

Information on likely routes of exposure

Inhalation

Skin contact

Eye contact

Ingestion

Acute toxicity

Not classified based on available information.

7 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

Product:

Acute dermal toxicity :  Acute toxicity estimate: > 5,000 mg/kg

Method: Calculation method

Components:

n-Heptane:

Acute oral toxicity :  LD50 (Rat): Expected > 5,000 mg/kg

Remarks: Information given is based on data obtained from

similar substances.

Acute inhalation toxicity :  LC50 (Rat, male and female): > 29.29 mg/l

Exposure time: 4 h

Test atmosphere: vapour

Method: OECD Test Guideline 403

Assessment: No adverse effect has been observed in acute

inhalation toxicity tests.

Acute dermal toxicity :  LD50 (Rabbit): Expected > 2,000 mg/kg

Assessment: No adverse effect has been observed in acute

dermal toxicity tests.

Remarks: Information given is based on data obtained from

similar substances.

Skin corrosion/irritation

Causes skin irritation.

Product:

Remarks: May cause skin irritation and/or dermatitis.

Components:

Heptane, branched, cyclic and linear:

Result: Irritating to skin.

n-Heptane:

Result: Irritating to skin.

Serious eye damage/eye irritation

Not classified based on available information.

Product:

Remarks: Vapours may cause irritation to the eyes, respiratory system and the skin.

Components:

Heptane, branched, cyclic and linear:

Result: Possibly irritating to eyes

8 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

n-Heptane:

Result: Possibly irritating to eyes

Respiratory or skin sensitisation

Skin sensitisation

Not classified based on available information.

Respiratory sensitisation

Not classified based on available information.

Components:

Heptane, branched, cyclic and linear:

Result: Substance is not considered to be potential skin sensitiser.

n-Heptane:

Test Type: Maximisation Test

Species: Guinea pig

Result: Did not cause sensitisation on laboratory animals.

Remarks: Information given is based on data obtained from similar substances.

Germ cell mutagenicity

Not classified based on available information.

Components:

n-Heptane:

Genotoxicity in vitro : Remarks: In vitro tests did not show mutagenic effects

Carcinogenicity

Not classified based on available information.

IARC No component of this product present at levels greater than or

equal to 0.1% is identified as probable, possible or confirmed

human carcinogen by IARC.

OSHA No component of this product present at levels greater than or

equal to 0.1% is on OSHA’s list of regulated carcinogens.

NTP No component of this product present at levels greater than or

equal to 0.1% is identified as a known or anticipated carcinogen

by NTP.

Reproductive toxicity

Not classified based on available information.

STOT - single exposure

May cause drowsiness or dizziness.

Components:

Heptane, branched, cyclic and linear:

Exposure routes: Inhalation

9 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

Target Organs: Central nervous system

Assessment: May cause drowsiness or dizziness.

n-Heptane:

Assessment: May cause drowsiness or dizziness.

STOT - repeated exposure

Not classified based on available information.

Aspiration toxicity

May be fatal if swallowed and enters airways.

Product:

May be fatal if swallowed and enters airways.

Components:

Heptane, branched, cyclic and linear:

May be fatal if swallowed and enters airways.

n-Heptane:

May be fatal if swallowed and enters airways.

Further information

Product:

Remarks: Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vom-

iting.

SECTION 12. ECOLOGICAL INFORMATION

Toxicity

Additional ecological :  An environmental hazard cannot be excluded in the event of

information unprofessional handling or disposal.

Very toxic to aquatic life with long lasting effects.

SECTION 13. DISPOSAL CONSIDERATIONS

Disposal methods

Waste from residues : Dispose of in accordance with all applicable local, state and

federal regulations.

Contaminated packaging : Empty remaining contents.

Dispose of as unused product.

Do not re-use empty containers.

Do not burn, or use a cutting torch on, the empty drum.

10 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

SECTION 14. TRANSPORT INFORMATION

Dangerous goods descriptions (if indicated below) may not reflect quantity, end-use, or region-specific

exceptions that can be applied. Consult shipping documents for descriptions that are specific to the

shipment.

International Regulations

IATA-DGR

UN/ID No. : UN 1993

Proper shipping name : Flammable liquid, n.o.s.

(Heptane)

Class : 3

Packing group : II

Labels : 3

Packing instruction (cargo : 364

aircraft)

Packing instruction : 353

(passenger aircraft)

IMDG-Code

UN number : UN 1993

Proper shipping name : FLAMMABLE LIQUID, N.O.S.

(Heptane)

Class : 3

Packing group : II

Labels : 3

EmS Code : F-E, S-E

Marine pollutant : yes

Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable for product as supplied.

National Regulations

49 CFR

UN/ID/NA number : UN 1993

Proper shipping name : Flammable liquids, n.o.s.

(Heptane)

Class : 3

Packing group : II

Labels : 3

ERG Code : 128

Marine pollutant : no

SECTION 15. REGULATORY INFORMATION

EPCRA - Emergency Planning and Community Right-to-Know Act

CERCLA Reportable Quantity

Components CAS-No. Component RQ Calculated product RQ

(lbs) (lbs)

Ethyl acetate 141-78-6 100 100 (F003)

Ethyl acetate 141-78-6 5000 *

11 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

*: Calculated RQ exceeds reasonably attainable upper limit.

SARA 304 Extremely Hazardous Substances Reportable Quantity

This material does not contain any components with a section 304 EHS RQ.

SARA 302 Extremely Hazardous Substances Threshold Planning Quantity

This material does not contain any components with a section 302 EHS TPQ.

SARA 311/312 Hazards :  Flammable (gases, aerosols, liquids, or solids)

Aspiration hazard

Skin corrosion or irritation

Specific target organ toxicity (single or repeated exposure)

SARA 313 :  This material does not contain any chemical components with

known CAS numbers that exceed the threshold (De Minimis)

reporting levels established by SARA Title III, Section 313.

California Prop. 65

WARNING: This product can expose you to chemicals including beta-Myrcene, which is/are

known to the State of California to cause cancer, and n-Hexane, which is/are known to the State

of California to cause birth defects or other reproductive harm. For more information go to

www.P65Warnings.ca.gov.

SECTION 16. OTHER INFORMATION

Further information

NFPA:

Flammability

3

Instability

2  0

Health

Special hazard.

Revision Date :  09/19/2019

The information provided in this Safety Data Sheet is correct to the best of our knowledge, infor-

mation and belief at the date of its publication. The information given is designed only as a guid-

ance for safe handling, use, processing, storage, transportation, disposal and release and is not

to  be  considered  a  warranty  or  quality  specification.  The information  relates  only  to  the  specific

12 / 13 
SAFETY DATA SHEET

Mi 6:8    SAPPHIRE TIRE DRESSING HIGH GLOSS

Dressing

Version  Revision Date:  SDS Number:  Date of last issue: 09/09/2019

5.0 09/19/2019 600000000718 Date of first issue: 05/23/2016

material  designated  and  may  not  be  valid  for  such  material  used  in  combination  with  any  other

materials or in any process, unless specified in the text.

US / EN

13 / 13